نام محصول : دوربین تحت شبکه  IP Camera

نوع محصول : Speed dome

سازنده : شرکت فناوری نوین مرصاد فاطر

مدل : SR-1663P-PA

| KEY FEACTURE |  |  |
| --- | --- | --- |
| ·Panoramic Camera ·Four 1/1.8" CMOS ·Up to 5520x2400@30fps ·Horizontal: 180°, Verticle:85° ·Min. illumination Color: 0.0004Lux@F1.0 ·Support region clipping ·Support distortion correction ·Detail Camera |  | ·Auto-Tracking Early Warning (AEW) ·Up to 2560*1440@30fps ·Optical zoom: 63x, digital zoom 16x ·Support 3d position ·Laser Distance: 500m ·Auto Wiper ·Built-in Wiper ·IP66 |
| Camera |  |  |
| Image Sensor | Panorama: 1/1.8" CMOS, Detail: 1/1.8" CMOS |  |
| Signal System | PAL/NTSC |  |
| Min. Illumination | Panorama: Color: 0.0004Lux@(F1.0, AGC ON), B/W: 0lux with IR |  |
| Shutter Time | 1s to 1/100,000s |  |
| Day & Night | IR Cut Filter with Auto Switch |  |
| Optical Zoom | Detail: 63x |  |
| Digital Zoom | Detail: 16x |  |
| Lens |  |  |
| Focal Length | Panorama: 2.8mm; Detail: 5.7-359mm |  |
| Zoom Speed | <5s |  |
| Field of View | Panorama: Horizontal: 180°, vertical: 85° Detail: Horizontal: 60° (w)~1.5° (t), vertical:35° (w)-0.9°(t) |  |
| Aperture Range | Panorama: F1.0; Detail: F1.6 ~ F5.0 |  |
| Focus Control | Auto/Manual |  |
| Iris Control | Auto |  |
| Close Focus Distance | Detail:1 m–3 m |  |
| Pan & Tilt |  |  |
| Pan Range | Detail: 0°~360° |  |
| Pan Speed | Panorama: Pan Manual Speed: 0.1°~200°/s°/s, Pan Preset Speed: 240°/s |  |
| Tilt Range | Detail: -21°-90° (Auto flip) |  |
| Tilt Speed | Detail: Pan Manual Speed: 0.1°~120°/s, Pan Preset Speed: 180°/s |  |
| 3D Position | Yes |  |
| Preset | 500 |  |
| Preset Accuracy | ±0.1° |  |
| Cruise | 16, up to 32 presets per cruise |  |
| Scan | 8 |  |
| Pattern | 8, each with 600s memory or 1000 orders |  |
| Scheduled Task | Preset, Cruise, Scan, Pattern, Alarm output |  |
| Park Action | Yes |  |
| Power-off Memory | Yes |  |
| Illuminator |  |  |  |
| IR LEDs | 11 |  |  |
| IR Distance | Up to 300m |  |  |
| IR Irradiation Angle | Adjustable by Zoom |  |  |
| White LEDs | 2, support white/blue/red light flash |  |  |
| Laser | 6, Wavelength 850nm |  |  |
| Laser Distance | Up to 500m |  |  |
| IR LEDs | 11 |  |  |
| IR Distance | Up to 300m |  |  |
| Compression Standard |  |  |  |
| Video Compression | S+265/H.265/H.264/M-JPEG |  |  |
| Video Bit Rate | 32Kbps~16Mbps |  |  |
| Audio Compression | G.711A/G.711U/ADPCM_D/AAC_LC |  |  |
| Audio Sampling Rate | 8kHz/32kHz/48kHz |  |  |
| Image |  |  |  |
| Max. Resolution | Panorama: 5520x2400;Detail: 2560x1440 |  |  |
| Main Stream | Panorama: PAL:25fps(5520x2400,4096x1800,3840x1680,2784x1200) NTSC:30fps(5520x2400,4096x1800,3840x1680,2784x1200) Detail: PAL:25fps(2560x1440,2304x1296,1920x1080,1280x720) NTSC:30fps(2560x1440,2304x1296,1920x1080,1280x720) |  |  |
| Sub Stream | Panorama: PAL:25fps(2048x896,1280x560,704x320) NTSC:30fps(2048x896,1280x560,704x320) Detail: PAL:25fps(704x576,704x288,352x288,640x360) NTSC:30fps(704X480,704X240,352X240,640x360) |  |  |
| Third Stream | Panorama: PAL:25fps (1280x560) NTSC:30fps (1280x560) Detail: PAL:25fps(704x576,704x288,352x288,640x480) NTSC:30fps(704X480,704X240,352X240,640x480) |  |  |
| BLC | Yes |  |  |
| HLC | Yes |  |  |
| WDR | Panorama :Digital; Detail: 120dB |  |  |
| Image Stabilization | Yes |  |  |
| Defog | Digital |  |  |
| White Balance | Yes |  |  |
| AGC | Yes |  |  |
| Noise Reduction | 2D/3D |  |  |
| ROI | 7 Dynamic Areas for Both Main and Sub Stream |  |  |
| Picture Overlay | Yes |  |  |
| Privacy Mask | Yes |  |  |
| Demist | Yes |  |  |
| Image Setting | Saturation, brightness, contrast, sharpness, adjustable by client software or web browser |  |  |
| Flip | Yes |  |  |
| OSD | Panorama: 16x16, 24x24,32x32, 48x48, 64x64, 96x96, 128x128, adaptable size, letters such as Week, Date, Time, Total 5 Region Detail: 16*16/24*24/32*32/48*48/64*64/72*72/96*96, adaptable size, letters such as Week, Date, Time, Total 5 Region |  |  |
| Feature |  |  |  |
| Alarm Trigger | Event Input, Event Output, Mask Alarm, Disk Full, Disk R/W Error, IP Address Conflict, MAC Address Conflict, FTP Server Exception |  |  |
| Video Analytics | Panorama: Tripwire, Double Tripwire, Heatmap Detail: Tripwire, Double Tripwire, Perimeter, Object Abandon, Object Lost, Loitering, Running, Parking, Heatmap, crowed, People Counting, On Duty Detection, Video Abnormal, Audio Abnormal, Face Capture, Human/Vehicle Classification |  |  |
| Early Warning (EW) | Yes |  |  |
| Auto-Tracking | Yes |  |  |
| Face Detection and Analytics |  |  |  |
| Operating Mode | Face capture, Intelligent monitoring |  |  |
| Face Detection Performance | Detects up to 32 faces at the same scene |  |  |
| Face Exposure Brightness | Default 35, 1~100 brightness optional |  |  |
| Min. Face Pixel | 29~2560 optional |  |  |
| Network |  |  |  |  |
| Protocols | HTTP/HTTPS/TCP/UDP/UPnP/SNMP/DHCP/DNS/DDNS/EasyDDNS/NTP/SMTP/SSL/802.1X/QoS/IPv4/IPv6/PPPOE/Unicast/Multicast/RTMP/RTSP/NFS/FTP |  |  |  |
| System Compatibility | ONVIF (PROFILE S/T/G/M), SDK, CGI, P2P |  |  |  |
| ANR | Auto store video in SD card when NVR disconnected, and upload video to NVR when resumed connection (Only Support Sorena ANR NVR) |  |  |  |
| Remote Connection | ≤7 |  |  |  |
| User/Host | Up to 32 users. 4 user levels |  |  |  |
| Web | IE, Firefox, Edge, Safari, Chrome |  |  |  |
| Interface |  |  |  |  |
| Communication Interface | 1 RJ45 100M/1000M self adaptive Ethernet port |  |  |  |
| Audio I/O | 1/1 |  |  |  |
| Alarm I/O | 8/2 |  |  |  |
| Built-In Speaker | Yes |  |  |  |
| Reset Button | Yes |  |  |  |
| On-board storage | Built-in Micro SD card slot, up to 512 GB |  |  |  |
| RS-485 | Yes |  |  |  |
| BNC | Yes(Debugging use) |  |  |  |
| Wiper | Yes |  |  |  |
| Electronic Compass | Yes |  |  |  |
| GPS/BDS | N/A |  |  |  |
| General |  |  |  |  |
| Protection | IP66, TVS 6000V lightning protection, surge protection and voltage transient protection |  |  |  |
| Operating Conditions | -40°C~70°C (IR OFF), -40°C~40°C (IR ON), 0-95%RH |  |  |  |
| Power Supply | DC36V±25% |  |  |  |
| Power Consumption | 76W MAX |  |  |  |
| Heater | Yes |  |  |  |
| Material | ADC 12 |  |  |  |
| Package/Boundary Dimensions | 565mm(L)*485mm(W)*750mm(H) /358mm(L)*358mm(W)*568mm(H) |  |  |  |
| Gross/Net Weight | 21.7kg (14.8kg) |  |  |  |
| Protection | IP66, TVS 6000V lightning protection, surge protection and voltage transient protection |  |  |  |
| Web Client Language | 17 languages Simple Chinese/ Traditional Chinese/ English/ Spanish/ Korean/ Italian/ Turkish/ Russian/ Thai/French/ Polish/ Dutch/ Hebraism/ Arabic/ Vietnamese/ German/ Ukrainian |  |  |  |
| Operating Conditions | -40°C~70°C (IR OFF), -40°C~40°C (IR ON), 0-95%RH |  |  |  |
| Power Supply | DC36V±25% |  |  |  |